| FORGE FORGEDBYARIS.COM | Rehab Punch List Checklist Final Walkthrough — Every Deficiency Documented |
| --- | --- |

Instructions: Walk the property with the contractor at completion. Check every item below. Mark ✓ for complete, or note the deficiency in the description column. No final payment is released until every item is signed off.

EXTERIOR

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | Exterior Body Paint Touch-Up | Walk all exterior walls. Confirm the paint is the correct color per the SOW and coverage is complete with no misses, drips, or thin spots. |
| ☐ | Fascia & Eaves — Termite Damage Repaired | Inspect all fascia boards and eaves for damage. Confirm any termite or rot damage identified in the SOW has been fully repaired and replaced. |
| ☐ | Fascia & Eaves — Paint Touch-Up | Confirm fascia and eaves are painted the correct color per the SOW with sufficient coverage. |
| ☐ | Fence Gates — Operation | Open and close all fence gates to confirm smooth operation — no sticking, rubbing, or misalignment. |
| ☐ | Pergola — Top Painted | Inspect the top of the pergola if accessible. Confirm it is painted the correct color. Confirm all bolts and hangers are painted. |
| ☐ | Exterior Lights Operational | Test all exterior light fixtures to confirm proper functionality. Confirm all fixtures have working bulbs installed. |
| ☐ | New Fencing — Painted or Clear Coated | Confirm all new fencing is finished per the SOW — painted the correct color or sealed with a clear coat. |
| ☐ | Deck — Painted or Clear Coated | Confirm the deck finish matches the SOW specification — painted or clear coated as called out. |
| ☐ | Exterior Doors — Paint Complete | Confirm all exterior doors are painted the correct color per the SOW with complete coverage and no drips. |
| ☐ | Exterior Doors — Open, Close, & Latch | Open and close all exterior doors. Confirm they latch properly and all hardware is installed correctly and securely. |
| ☐ | No Exterior Plumbing Leaks | Inspect all exterior plumbing — irrigation valves, hose bibs, and any exposed lines. Confirm no visible leaks. |

LANDSCAPING

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | No Dead Spots in Grass | Walk all turf areas and confirm there are no dead or brown spots. Any dead grass must be replaced before sign-off. |
| ☐ | Sprinklers Operational & Directed Correctly | Run the sprinkler system through all zones. Confirm all heads are working, adjusted correctly, and not spraying sidewalks, the house, or fencing. |
| ☐ | No Dead Plants | Inspect all planted areas for dead or dying plant material. Replace before final sign-off. |
| ☐ | Bark / Ground Cover Evenly Spread | Confirm ground cover in all planter areas is spread evenly with consistent depth and no bare spots. |
| ☐ | Yards Clean & Presentable | Confirm all yard areas are clean, free of debris, and present at a listing-ready standard. |

COMMON LIVING SPACES & HALLWAYS — CEILINGS

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | Ceiling Texture — Complete & Uniform | Walk each room and inspect ceiling texture. Confirm complete coverage with no thin spots, gaps, or visible patches. |
| ☐ | Ceiling Drywall Patches — Invisible | Inspect all patched areas. Patches must be fully feathered and not visible after texture and paint. |
| ☐ | Ceiling Paint Touch-Up — Complete | Confirm ceiling paint is complete with no misses, uneven sheen, or visible roller lines. |

COMMON LIVING SPACES & HALLWAYS — WALLS

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | Wall Texture — Complete & Uniform | Inspect wall texture in all rooms. Confirm consistent application with no visible patching, missed areas, or texture mismatches. |
| ☐ | Wall Drywall Patches — Invisible | Inspect all wall patches. Must be fully finished and not visible after texture and paint. |
| ☐ | Wall Paint Touch-Up — Complete | Confirm all wall paint is complete — no misses, drips, roller lines, or color inconsistencies. |

COMMON LIVING SPACES — ELECTRICAL

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | Light Switches — Installed Correctly | Test all light switches. Confirm switches are not loose or wobbly and all operate correctly. |
| ☐ | Outlets — Installed & Wired Correctly | Use an outlet tester on every outlet. Confirm all are wired correctly, functional, and not loose. Half-hot outlets installed upside down per code. |

CABINETS

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | Cabinets Open & Close Properly | Open and close every cabinet door and drawer. Confirm smooth operation, proper alignment, and no rubbing or sticking. |
| ☐ | Cabinets Painted Correctly — Interior & Exterior | Inspect all painted cabinet surfaces. Confirm correct color, complete coverage, and clean interior paint. |
| ☐ | Cabinet Hardware Installed Correctly | Confirm all pulls, knobs, and hinges are the correct hardware per the SOW and are installed straight and securely. |

DOORS & CASING — COMMON AREAS

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | Door Casing — Caulked & Nail Holes Filled | Inspect all door casing. Confirm all nail holes are filled, casing joints are caulked with no visible seams, and no gaps remain at walls. |
| ☐ | Door Casing — Painted Correctly | Confirm all door casing is painted the correct color with complete coverage. |
| ☐ | Doors — Open, Close, & Latch | Open and close every interior door. Confirm doors do not stick on the jam or rub on the floor. Confirm all latches engage. |
| ☐ | Door Hardware — Installed Correctly | Inspect all door knobs, levers, and locks. Confirm correct hardware per SOW, installed securely and functioning properly. |
| ☐ | Slider Screens — Present & Functional | Confirm screen doors are present on all sliding doors. Test operation and confirm no rips in screen material. |

BASEBOARDS

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | Baseboards — Caulked Completely | Inspect all baseboards. Confirm caulking is applied at all wall-to-baseboard seams with no gaps visible. |
| ☐ | Baseboards — Nail Holes Filled & Painted | Confirm no visible nail heads in any baseboard. All holes filled and painted over. |
| ☐ | Baseboards — Painted & Joints Caulked | Confirm correct paint color with complete coverage. All joints at corners and transitions are caulked with no visible seams. |

WINDOWS — COMMON AREAS

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | Windows — Open, Close & Lock | Open, close, and lock every window. Confirm smooth operation and that locking mechanisms engage fully. |
| ☐ | Window Screens — Present & Undamaged | Confirm screens are present on all windows, installed correctly, and have no rips or damage. |
| ☐ | Windows — Caulked & Sealed | Inspect the perimeter of all window frames. Confirm no gaps where water can infiltrate. |

HARDWOOD FLOORS

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | New Hardwood — No Glue in Seams | Inspect all seams in new hardwood. No glue should be visible seeping through. |
| ☐ | New Hardwood — Joints Tight | Confirm no visible gaps between hardwood boards. Any gaps indicate improper installation. |
| ☐ | New Hardwood — T-Bar Moldings Secured | Confirm all transition T-bar moldings between wood and tile are glued down, secured, and no glue is visible. |
| ☐ | Existing Hardwood — Gaps Filled | Inspect existing hardwood for gaps or voids. Confirm all have been filled with the correct wood filler. |
| ☐ | Existing Hardwood — Sealed & Stained | Confirm the floor has been stained the correct color per the SOW and sealed completely — no raw or unsealed areas. |

BATHROOMS

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | Toilet — Operational | Flush each toilet. Confirm it flushes fully, fills correctly, and has no running or rocking at the base. |
| ☐ | Toilet — Seat & Hardware | Confirm toilet seat is installed, hinges are tight, and all hardware is present and secured. |
| ☐ | Sink & Faucet — Functional | Run hot and cold water. Confirm proper pressure, temperature separation, and no drips at faucet or under sink. |
| ☐ | Tub Stopper — Operational | Test the tub stopper. Confirm it creates a proper seal and releases correctly. |
| ☐ | Tub Overflow — No Leaks | Fill tub above the overflow line. Confirm the overflow operates correctly and there are no leaks at the overflow plate. |
| ☐ | Tub Drain — Drains Properly | After filling the tub, confirm it drains completely with no slow-drain issues. |
| ☐ | Bath & Shower Tile — Correct & Secure | Confirm correct tile type and pattern per SOW. Inspect for loose, broken, or chipped tiles — replace any defects. |
| ☐ | Grout — Filled & Color Correct | Inspect all grout lines. Confirm correct color per SOW, complete fill with no gaps or voids. |
| ☐ | Shower Surround Seam — Silicone Caulked | Inspect the seam between the shower surround and the tub. Confirm silicone caulk is applied, no gaps or voids, and finish is clean. |
| ☐ | Floor Tile — Installed Correctly | Confirm correct tile pattern and grout per SOW. Inspect for loose, broken, or chipped tiles. |
| ☐ | Bathroom Doors — Functional | Open, close, and latch all bathroom doors. Confirm hardware is installed correctly and securely. |

GARAGE

| ✓ | ACTION ITEM | DESCRIPTION / NOTES |
| --- | --- | --- |
| ☐ | No Open Electrical Splices | Inspect all garage electrical for exposed splices or junction boxes without covers. All splices must be in covered junction boxes. |
| ☐ | All Outlets Functional | Use an outlet tester on all garage outlets. Confirm proper wiring and functionality. |
| ☐ | All Lights Functional with Bulbs | Turn on all garage lights. Confirm all fixtures have bulbs installed and are working. |
| ☐ | Garage Door Opener — Functional | Open and close the garage door using the opener. Test floor sensors for proper safety stop function. |
| ☐ | Garage Door Opener — Plugged Directly to Outlet | Confirm the garage door opener is plugged directly into a wall outlet — not an extension cord. |
| ☐ | Firewall Holes Patched | Inspect the wall between the garage and the house. Confirm all penetrations are patched with the correct fire-rated drywall and there are no holes. |
| ☐ | Fire-Rated Door — Installed with Self-Closing Hinges | Confirm the correct fire-rated door is installed between the garage and the house. Self-closing hinges (bomber hinges) must be installed and the door must fully close on its own. |

SIGN-OFF

| Property Address: |  | Date: |  |
| --- | --- | --- | --- |

| Contractor Signature: |  | Date: |  |
| --- | --- | --- | --- |

| Owner / Project Manager Signature: |  | Date: |  |
| --- | --- | --- | --- |

FORGE  |  Powered by ARIS  |  ForgedByAris.com